1：下位机连接PC后，HID号为

After connect the lower computer to PC, HID will be

USB_VID         0x0416

USB_PID         0x5020

HID_CMD_SIGNATURE   0x43444948

2：打开设备连接后，上位机读取下位机IO状态（此状态为16BIT数，每一位表示一个状态，最低位为1号IO）显示在上位机上

2 . Open device connection, the Host computer read lower computer IO state (This state is 16BIT, each byte stands for one state, the lowest is number 1, IO )

3：上位机控制命令是：  第一路IO开：0X0001     第一路IO关是：0X0000

第二路IO开：0X0002     第二路IO关是：0X0000

第三路IO开：0X0004     第三路IO关是：0X0000

.

The control command of	 the host computer:

Open the first relay IO：0X0001, Close the first relay IO：0X0000

Open the second relay IO：0X0002, Close the second relay IO：0X0000

Open the third relay IO：0X0004, Close the third relay IO：0X0000

.

第一、二、三路同时开：0x0007

关第二路，开第一、三路：0x0005

Open the first three relays simultaneously:. 0x0007

.Close the second one, open the first and third one: 0x0005

.

每一个bit代表一个IO状态，1为开，0为关！！

Each bit stands for IO state, 1 for open, 0 for close!

- 命令测试工具 CommandApp_USBRelay.exe 使用说明：

- The instructions of Command test tool “CommandApp_USBRelay.exe”

命令格式：CommandApp_USBRelay [设备序列号]  [操作类型]  [继电器开关]

command format: CommandApp_USBRelay [Inmarsat Serial Number] [Operation type] [Relay switch]

[参数说明] [parameter specification]

设备序列号：每个设备有一个唯一的序列号，可以通过GuiApp_Chinese.exe或者GuiApp_English.exe程序获取

操作类型：  open--打开继电器， close-- 关闭继电器

继电器开关：01表示第一个继电器，02表示第二个继电器，... 255表示所有继电器动作

Inmarsat Serial Number: Each device has only one serial number, it can be gotten form GuiApp_Chinese.exe or GuiApp_English.exe

Operation type: open--open the relay, close-- close the relay

Relay switch: 01 stands for the first relay, 02 stands for the second relay... 255 stands for all relays

例如：For instance

- 1. 打开序列号afEd5设备的第一路继电器Open the first relay of series number of Device afEd5

CommandApp_USBRelay  afEd5 open 01

- 2. 打开序列号afEd5设备的所有继电器Open all relays of series number of Device afEd5

CommandApp_USBRelay  afEd5 open 255

- 3. 关闭序列号afEd5设备的第一路继电器Close the first relay of series number of Device afEd5

CommandApp_USBRelay  afEd5 close 01

- 4. 关闭序列号afEd5设备的所有继电器Close all relays of series number of Device afEd5

CommandApp_USBRelay  afEd5 close 255

命令行工具执行完命令后，如果没有错误，则返回0， 否则返回 1

调用者可以根据该值判断执行是否正确

After executing the above command of command line tools, if all is correct, it will be back to 0. Otherwise, it will be 1.

- 二、GuiApp_Chinese.exe或者GuiApp_English.exe 是带界面的测试程序

先使用“查找设备”按钮查询设备，然后从下拉列表框选择你需要操作的设备，然后点击“打开设备”，即可进行操作

GuiApp_Chinese.exe and GuiApp_English.exe are the test program with interface.

Looking for devices by “scan for devices”, then choosing the device you need to operate from drop-down list box. And click “scan for devices”.