# The Instruction of 0-30V Adjustable voltage / regulated power supply

This is a high-quality continuous adjustable power supply and the input voltage is 0-30v. Current limiting circuit also includes in Output voltage. This power supply can effectively control the output constant current from 2mA to 3A. The advantage of this regulated power supply is necessary for most of experiment circuit. You can switch the power supply without worrying about damaging the experiment circuit when mistaken-operating, because this regulated power supply can control the maximum current.

Technical Specification as follow,

Input Voltage: AC24V

Max Input Current: 3A

Output Voltage:0-30V in adjustment

Output Current：2mA-3A in adjustment

# Max Output voltage ripple：0.01%

Circuit characteristic：

1.All are thru-hole components, convenient for installation and maintenance

2.Easily adjust the output voltage。

3.LED display the limited output current.

4. Provide overcurrent fault protection.

Component list:

First step

Install the resistance, the diode and other component as the PCB indicates. Please make sure resistance is correct with the multimeter, and the diode should be in the correct model and installing direction.

Install other component in accordance with the  principle that from largest to smallest, lowest to highest.

The potentiometer can be installed directly on the board, or with socket and wires. You may also replace it with multiturn potentiometer, which makes adjustment more precise.

After installing all components, and checked them out without problem, you may power on and take a trial run. Please make sure Q4(D1047) has enough big heat sinks before power on. If the heat sinks is not big enough, you may cool it with extra fan. There is a 24V port for the fan on the board, and the heat sinks should be insulated form the circuit.

Circuit Adjustment：

- Adjust the potentiometer(the green one indicated V) to the smallest position value(the smallest position value anticlockwise) with changing voltage. Adjust RV1, and make output voltage to OV. (It may appear negative voltage or the value is very low during adjusting. Please use it with digital multimeter). It is unnecessary to sdjust the maximum output voltage. When using 24 AC input, the output is about 30.3V.

- Calibrated current: Use the external load resistor 10Ω(ensure it has adequate power), Add the voltage to 1V slowly, adjust the Current-Limited potentiometer(the green one indicated A) to luminescent tube to shine. At this time, the limiting current of the circuit is 0.1A, please note down the position of of the potentiometer. Adjusting it to 2V, 5V, 10V, 20V, 30V and other values, then get their corresponding output current. The calculation formula is: I=U/R=U/10=0.1U. 10Ωloaded, when U is 30V, I=3A(maximum output). You may also change it with other load resistance in different resistance value only if it has adequate power and been cooled enough.

Attention:

- The transformer uses more than 80W, single 24V or dual 12V(the center of the head unoccupied). If the voltage decrease dramatically when using high-power output, then it needs replaced by the transformer that offering more power.

- Output control tube D1047 needs be insulated from circuit when installing the radiator. It is because this circuit is linear stabilized voltage supply, if it works with low voltage and high current for long time, the voltage of the control tube decrease a lot. Please make sure D1047 is well-cooled.

- Power making is the circuit with heavy current and high voltage. Please double check the components model and make sure it is installed correctly.